MurlokVR

Two-Process Shared Memory Architecture

Technical Architecture & Driver Documentation

Version 1.0 — Phase 1 (3DOF)

Author: Blockmann2K

Language: „Rust“ (Primary) + „C++“ (OpenVR Adapter)

1. Overview

MurlokVR Is a Custom DIY VR Headset With Inside-Out Tracking. The Software Architecture Is Designed Around a Two-Process Model That Allows Nearly All Logic To Be Written in Rust While Still Maintaining Full Compatibility With SteamVR, Which Requires Its Drivers To Be Written in C++.

This Document Covers the Complete Technical Architecture of the SteamVR Driver System, Including Process Responsibilities, the Shared Memory Bridge, Synchronization Strategy, Data Flow, and Future Scalability.

1.1. The Core Problem

SteamVR Mandates That Hardware Drivers Be Implemented Using the OpenVR C++ API. Building the Entire Driver in C++ Would Mean Abandoning Rust As the Primary Language, Losing Its Safety Guarantees, and Working Outside the Developer’s (My) Primary Skill Set.

1.2. The Solution

Two Independent Programs Run in Parallel and Communicate via a Shared Memory IPC Bridge:

- Process 1: VR Runtime (Rust) — The Real Brain of the System. Handles All Sensor Processing, Tracking, and State Management.

- Process 2: OpenVR Driver (C++) — A Minimal Adapter That Reads Shared Memory and Feeds Pose Data Into SteamVR.

Design Philosophy "Rust Does the Thinking. C++ Just Delivers the Mail." The C++ Driver Should Contain Zero Tracking Logic! If Real Logic Ever Appears on the C++ Side, It Should Be Pushed Back Into Rust Immediately!

2. Architecture Diagram

The Diagram Shows the Complete Data Pipeline From Hardware Sensor to SteamVR Frame:

[ ESP32 + BNO085 ]

│

│  USB-C (Serial)

▼

┌─────────────────────────────┐

│    VR Runtime (Rust)        │  ← The Brain

│                             │

│  - Serial Communication     │

│  - Sensor Fusion            │

│  - Pose Calculation         │

│  - State Management         │

│  - Heartbeat Stamping       │

└─────────────┬───────────────┘

│

│  Shared Memory (IPC Bridge)

│

┌─────────────▼───────────────┐

│    OpenVR Driver (C++)      │  ← The Postman

│                             │

│  - Reads Shared Memory      │

│  - Checks Heartbeat         │

│  - Feeds Poses → SteamVR    │

│  - Graceful Degradation     │

└─────────────┬───────────────┘

│

│  OpenVR API

▼

[ SteamVR ]

3. Process 1 — VR Runtime (Rust)

The VR Runtime Is the Core of the MurlokVR Software Stack. It Owns All Real Intelligence and Is Entirely Written in Rust. It Runs as an Independent Process That Can Be Started, Stopped, and Iterated On Without Touching SteamVR.

3.1. Responsibilities

- Serial Reading — Pulling Raw IMU Data Packets From the ESP32 Over USB-C Serial.

- Sensor Processing — Receiving the BNO085 Quaternion Output and Converting It Into a Clean, Usable Headset Pose.

- Pose Validation — Sanity-Checking Incoming Data Before Writing It to Shared Memory.

- State Management — Tracking Device Connection State: Connected, Calibrated, Lost, Disconnected.

- Shared Memory Writing — Continuously Writing the Latest Pose and State Into the Shared Memory Block With Atomic Synchronization.

- Heartbeat Updating — Stamping a u64 Timestamp on Every Write Cycle So the C++ Side Can Detect a Dead Process.

3.2. Key Advantage

Because the Rust Runtime Is Completely Separate From SteamVR, a Crash or Panic on the Rust Side Does NOT Crash SteamVR. The C++ Driver Will Detect the Stale Heartbeat and Gracefully Report the Device As Disconnected Instead.

3.3. Iteration Workflow

During Development, the Rust Runtime Can Be Restarted Freely Without Needing To Restart SteamVR. This Makes Iteration Significantly Faster Compared to an In-Process DLL Architecture (FFI).

4. Process 2 — OpenVR Driver (C++)

The OpenVR Driver Is Intentionally Kept As Thin and Dumb As Possible. Its Sole Purpose Is To Act as a Bridge Between the Shared Memory Block and the SteamVR API. It Contains No Tracking Logic Whatsoever.

4.1. Responsibilities

- Shared Memory Opening — At Startup, Opens the Named Shared Memory Block Created by the Rust Runtime.

- Heartbeat Checking — Before Every Read, Verifies the Rust-Side Timestamp Is Recent. If Stale, Reports Device As Disconnected.

- Atomic Lock Checking — Reads the Sequence Counter To Ensure No Write Is in Progress Before Copying Pose Data.

- Pose Forwarding — Translates the Shared Memory Data Into SteamVR’s ITrackedDeviceServerDriver Pose Format and Submits It.

- Graceful Degradation — If the Rust Process Dies or the Heartbeat Expires, Reports the HMD As Disconnected Rather Than Feeding SteamVR Invalid Data.

4.2. OpenVR Sample Reference

Valve Provides Official OpenVR Driver Samples That Serve As the Foundation for the C++ Adapter.

The Relevant Samples Are Located At:

https://github.com/ValveSoftware/openvr/tree/master/samples/drivers/drivers

4.3. Registering the Driver With SteamVR

After Building the Driver DLL, It Must Be Registered With SteamVR Using: vrpathreg.exe

The Output Directory Should Contain:

- bin/win64/driver_MurlokVR.dll

- resources/driver.vrresources

- driver.vrdrivermanifest

5. The Shared Memory Bridge

The Shared Memory Block Is the Contract Between the Two Processes. It Must Be Treated With Extreme Care — Every Field Must Have Identical Binary Layout on Both Sides. On the Rust Side, Every Struct That Crosses This Boundary Must Be Annotated With #[repr(C)].

5.1. Shared Memory Block Layout (Not Final)

| Field | Type (Rust) | Type (C++) | Purpose |
| --- | --- | --- | --- |
| Sequence Counter | AtomicU32 | std::atomic<uint32_t> | Atomic Lock |
| Quaternion X | f32 | float | Head Rotation Component X |
| Quaternion Y | f32 | float | Head Rotation Component Y |
| Quaternion Z | f32 | float | Head Rotation Component Z |
| Quaternion W | f32 | float | Head Rotation Component W |
| Heartbeat Timestamp | u64 | uint64_t | Unix Timestamp in „ms“ — Last Rust Write |
| Device Status | u32 | uint32_t | 0 = Disconnected, 1 = Connected, 2 = Calibrated, 3 = Lost |
| (Phase 2) Position X | f32 | float | Head Position X — 6DOF Addition |
| (Phase 2) Position Y | f32 | float | Head Position Y — 6DOF Addition |
| (Phase 2) Position Z | f32 | float | Head Position Z — 6DOF Addition |

Critical Rule: #[repr(C)] Every Struct Written to Shared Memory on the Rust Side MUST Be Annotated With #[repr(C)]. Without This, Rust Is Free To Reorder Struct Fields However It Chooses at Compile Time. The C++ Side Would Then Silently Read Garbage Data With No Error or Warning.

6. Synchronization — Atomic Sequence Lock

Two Processes Accessing the Same Memory Region Simultaneously Without Coordination Will Produce Torn Reads — for Example, Reading a Quaternion Where Half the Floats Belong to the Previous Frame and Half to the Current Frame. This Results in Garbage Rotation Data Being Sent to SteamVR.

MurlokVR Uses an Atomic Sequence Lock To Coordinate Access Without Requiring OS-Level Locking, Keeping Latency Minimal and the Implementation Straightforward.

6.1. How the Atomic Sequence Lock Works

The Sequence Counter Is a Single 32-Bit Atomic Integer Shared in the Memory Block. The Protocol Is:

- Before Writing, the Rust Side Increments the Counter to an ODD Number.
-> This Signals: Write in Progress.

- The Rust Side Writes All Pose Data Fields to Shared Memory.

- After Writing, the Rust Side Increments the Counter Again to an EVEN Number.
-> This Signals: Write Complete.

- The C++ Side Reads the Counter.
-> If It Is ODD, a Write Is in Progress — Skip This Frame, Try Again Next Poll.

- If the Counter Is EVEN, the C++ Side Reads the Full Pose.
-> It Then Re-Reads the Counter To Confirm It Hasn’t Changed During the Read.
-> If It Has Changed, Discard and Retry!

Why Not a Mutex? A Named Mutex (Option B) Is Safer but Introduces OS-Level Scheduling Overhead and Cross-Language Setup Complexity! For a 90Hz VR Pipeline Where Every Millisecond Counts, the Lock-Free Atomic Sequence Lock Is the Standard Choice and Keeps Things Simpler To Implement. ;)

6.2. Sequence Lock Protocol Summary

| Step | Actor | Counter State | Action |
| --- | --- | --- | --- |
| 1 | Rust (Writer) | Even → Odd | Increment Counter (Signals Write Starting) |
| 2 | Rust (Writer) | Odd | Write Quaternion + Timestamp + Status |
| 3 | Rust (Writer) | Odd → Even | Increment Counter (Signals Write Complete) |
| 4 | C++ (Reader) | Even | Read Counter, Proceed With Pose Read |
| 5 | C++ (Reader) | Even | Read All Pose Fields From Shared Memory |
| 6 | C++ (Reader) | Even (Verify) | Re-Read Counter — If Changed, Discard & Retry |
| 7 | C++ (Reader) | Even (Confirmed) | Submit Pose to SteamVR |

7. Heartbeat & Crash Safety

A Critical Safety Feature of the Two-Process Architecture Is That a Crash or Freeze in the Rust Runtime Must Never Cause SteamVR To Receive Invalid Pose Data Indefinitely. The Heartbeat Mechanism Ensures This.

7.1. How the Heartbeat Works

- The Rust Runtime Writes the Current System Timestamp (in Milliseconds) to the Heartbeat Timestamp Field on Every Write Cycle.

- The C++ Driver Reads This Timestamp Before Every Pose Submission to SteamVR.

- If the Timestamp Is Older Than a Defined Threshold (e.g., 500ms), the Rust Process Is Considered Dead or Frozen.

- In This Case, the C++ Driver Reports the HMD As Disconnected to SteamVR via IVRServerDriverHost Rather Than Forwarding Stale Poses.

7.2. Recommended Heartbeat Threshold

| Scenario | Recommended Threshold | Behavior on Stale |
| --- | --- | --- |
| Normal 90Hz Operation | < ~10ms Between Writes | No Action — Within Normal Operation |
| Transient Stall / USB Hiccup | < ~100ms Threshold | Freeze Last Known Good Pose for 1–2 Frames |
| Rust Process Crashed / Hung | > ~500ms Threshold | Report HMD As Disconnected to SteamVR |

Crash Isolation Advantage This Is One of the Key Benefits of the Two-Process Architecture Over the In-Process DLL Approach. If the Rust Runtime Panics, SteamVR Keeps Running Cleanly. The User Sees the Headset Disconnect In SteamVR Rather Than a Full SteamVR Crash. The Rust Process Can Be Restarted Independently.

8. Data Flow — One Full Frame

The Following Describes the Complete Lifecycle of a Single Pose Update From Hardware to SteamVR:

| Step | Actor | Description |
| --- | --- | --- |
| 1 | BNO085 (IMU) | Sensor Fusion Hardware Fires a New Rotation Quaternion at Its Configured Output Rate. |
| 2 | ESP32 | Microcontroller Reads the Quaternion From BNO085 via I2C and Packages It Into a Serial Packet. |
| 3 | USB-C Serial | Serial Packet Is Transmitted Over USB-C to the Gaming Laptop. |
| 4 | VR Runtime (Rust) | Rust Runtime Reads the Serial Packet From the OS Serial Port Buffer. |
| 5 | VR Runtime (Rust) | Rust Validates and Processes the Quaternion. Updates Device Status if Needed. |
| 6 | VR Runtime (Rust) | Rust Performs Atomic Sequence Lock Write: Increment (Odd) → Write Pose → Increment (Even) → Stamp Heartbeat. |
| 7 | OpenVR Driver (C++) | C++ Driver Polls Shared Memory Every ~10ms (90Hz). Checks Heartbeat Freshness. |
| 8 | OpenVR Driver (C++) | C++ Reads Sequence Counter. If Even, Reads Pose. Re-Validates Counter After Read. |
| 9 | OpenVR Driver (C++) | C++ Constructs a DriverPose_t Struct and Calls IVRServerDriverHost::TrackedDevicePoseUpdated(). |
| 10 | SteamVR | SteamVR Receives the Pose, Updates the Virtual HMD Transform, and Renders the Next Frame. |

9. Startup & Shutdown Lifecycle

Process Startup and Shutdown Ordering Requires Deliberate Handling. Both Processes Need To Behave Gracefully Regardless of Which One Starts or Stops First.

9.1. Startup Sequence

- The Rust Runtime Starts First and Creates the Named Shared Memory Region.

- Rust Initializes All Fields to Safe Defaults:
-> Status = Disconnected, Quaternion = Identity (0, 0, 0, 1), Heartbeat = Current Time.

- The C++ OpenVR Driver Is Loaded by SteamVR. It Opens the Named Shared Memory Region.

- If the Rust Runtime Is Not Yet Running When the C++ Driver Loads, the C++ Side Should Wait/Poll for the Shared Memory To Become Available Rather Than Crashing.

9.2. Shutdown Sequence

- User Closes SteamVR: The C++ Driver Is Unloaded. The Rust Runtime Should Detect the Driver Is Gone (Optional) and Enter Standby Mode or Shut Down Cleanly.

- Rust Runtime Is Manually Stopped: The Heartbeat Goes Stale. The C++ Driver Detects This and Reports the HMD As Disconnected. SteamVR Continues Running.

- Rust Runtime Crashes: Identical Behavior to Manual Stop — Heartbeat Detection Handles It.

Who Owns the Shared Memory? The Rust Runtime Creates the Shared Memory Region. It Is the Owner. The C++ Driver Only Opens (Not Creates) the Region. On Windows, the Region Is Released Automatically When the Last Handle Is Closed.

10. Phase Scalability

The Two-Process Architecture Was Designed With MurlokVR’s Three-Phase Roadmap in Mind. Each Phase Requires Changes Primarily on the Rust Side, With Only Minor Updates Needed to the C++ Adapter.

| Phase | Tracking Type | New Shared Memory Fields | C++ Changes Required | Rust Changes Required |
| --- | --- | --- | --- | --- |
| Phase 1 (Current) | 3DOF Rotational | Quaternion (X, Y, Z, W) | Minimal — Submit Rotation-Only Pose | Serial Read, Quaternion Write, Heartbeat |
| Phase 2 | 6DOF Positional (SLAM) | Position (X, Y, Z) Added | Expose Position to SteamVR Pose Struct | Add SLAM Pipeline, Camera Processing, Position Write |
| Phase 3 | 6DOF Controllers | Per-Controller Structs Added | Register Controller Devices, Forward Inputs | Controller Tracking, Button State, Haptics |

Because the C++ Side Is Purely a Relay, the Evolution of MurlokVR’s Capabilities Lives Almost Entirely in Rust. This Is the Intended Design — Maximum Rust, Minimum C++. Yay!

If You Read This You Like Hatsune Miku!